| ## CHECKLIST PROJECT TERRACE ventilated |  |  |  |
| --- | --- | --- | --- |
| ☐ TILES |  | ☐ DECKING |  |
| Basic information |  |  |  |
| Investor |  |  |  |
| Address realization |  |  |  |
| Floor |  |  |  |
| Elevator | YES | NO | Consider that no elevator and the need to bring the material may be an additional expense |
| Bringing | YES | NO |  |
| Artist terrace | Editing self | Outsourcing company |  |
| details contact the company that installed the terrace |  |  |  |
| Expected date execution of the installation |  |  |  |

| Detailed information about the terrace |  |  |  |
| --- | --- | --- | --- |
| Purpose of the terrace | Recreational / public / fountain / restaurant / SPA (jacuzzi) / internal / technical other |  |  |
| Area m2 |  |  |  |
| Fixed 0 point of the terrace | Lower window frame / floor level inside other |  | Most often the zero point of the terrace is the bottom window frame or the level floors inside the room adjacent to the terrace. |
| Substrate type under pedestals | Styrodur XPS, heat-sealable roofing felt, liquid waterproofing, concrete, tiles, cubes, concrete blocks,native soil |  | Styrodur XPS300 is the recommended hardness for positioning the pedestals. |
| Stability and evenness of the substrate |  |  | The substrate should provide stable support for the pedestals to avoid excessive bending of the terrace during use. |
| Damage-Damage- |  |  | sensitive substrate sensitive substrate, such as liquid waterproofing, should be additionally secured with a SBR 200x200 rubber pad under the bracket. |
| Possibility of anchoring the pedestals to the ground | YES | NO | The pedestals cannot be anchored on the ground with waterproofing functions. |
| Declines on the terrace |  |  |  |
| Minimum height |  |  | The lowest adjustable support DD PEDESTALS has a height of 10 mm. |
| Maximum height The |  |  | highest adjustable bracket DD PEDESTALS is 950 mm high. |
| Laying the terrace | horizontally | In a slope | A terrace made of plates arranged horizontally or with a minimum slope of 0.5-1.0% looks the best. |
| Finishing of the terrace from the outside |  |  | If the terrace has exposed edges, it is necessary to provide for the installation of the masking frames. |

| Information for terrace BOARDS |  |  |  |
| --- | --- | --- | --- |
| size plate |  |  |  |
| thickness of the plate |  |  |  |
| Manufacturer plate |  |  |  |
| Model / name plate |  |  |  |
| Is the disc keeps the same thickness | YES | NO | If you select NI is recommended to use rubber pads SH to bridge the gap plates |
| Manufacturer's recommendation for board support pedestals | only in the corners / In the corners and in the middle / other …………………… .. |  | Each panel manufacturer should provide information whether they allow point support of the panels and on what conditions |
| The gap between the panels | 3 mm / 5 mm |  | 3 mm recommended for panels with an even edge, 5 mm recommended for boards with slight edge line deviations (then it is less visible). Water will flow freely from the terrace already at a width of 3 mm. |

| Information for the PLANK TERRACE |  |  |
| --- | --- | --- |
| Plank type | Wood, WPC composite other |  |
| Plank profile Plank |  |  |
| type | stoneware, concrete, stone other |  |
| Board manufacturer's recommendations regarding the distance between the |  |  |
| Joist type | Wood, composite, aluminum Other |  |
| Joist profile Joist |  |  |
| Joists manufacturer's recommendations regarding the distance between supports on the supports |  |  |

To The list should be attached with a drawing of the terrace

Checklist what should be included in the drawing: ☐ All terrace dimensions have been marked ☐ The terrace has edges shown. ☐ The lowest and highest point of the terrace ☐ Slopes on the terrace have been drawn ☐ Water drains have been marked ☐ Stairs and changes in the height of the terrace surface have been marked ☐ The necessary revisions ☐ Needed point where extra support is needed (eg. jacuzzi bath etc.) ☐ Installations under the terrace (electric, hydraulic pipes etc.)